File Preparation_

+ Open the template file and your working file.

+ Do not alter the size of the template

+ Copy your work from your file to the template.

+ Place only the work being cut/scored inside the bounding box and delete everything else.

+ Place your work on the correct cutting and scoring layers.

+ Check ALL of your layers and verify the lines properties are the correct color and size.

+ All lines must have the hairline width or they will not cut.

+ All geometry must be drawn on the construction plane ( X , Y Coordinates )

In the command bar type ProjectToCPlane.

When prompted to delete input object type Y, Enter.

The selected objects should be projected onto the 0 plane ( X, Y Coordinates ).

+ No duplicate geometry can be submitted.

In the command bar, type SelDup, Enter.

Delete selected objects.

+ All geometry must be curves or hatches.

+ Rasterized geometry must be hatched on appropriate layers.

+ All vector and hatched geometry must be Ungrouped.

Select all objects.

In the command bar, type ungroup, Enter.

+ All vector and hatched geometry must be exploded.

Select all objects.

In the command bar, type explode, Enter.

+ Colors must be true cyan, magenta, or yellow, or red, green or blue (255 in the color chooser for that color)

+ All files and materials much be positioned in the upper left hand corner of the template.

+ If you supply 3″ or 4″ by 22″ materials, they must be taped together

+ If you want your small parts kept in place you must supply your own tape.

+ Files waiting for materials will NOT be in line to be cut until the material is received and numbered.

+ The file must be submitted using either the 24”x36” or 18”x32” templates.

+ Instructions for Online Printing

- Sign in to the system using your eRaider username and password.

- Click the "Add Job" link to the left.

- Enter in the information to match the file you would like to print.

- Enter in your 4-digit course number. If file is not for a course enter 1111.

- Click Submit only once.

- On the next page you will need to upload a file.

- You will need to browse for your file and then click Upload only once.

- You will then be prompted to Add another job if you have more than one file to send.